TRUE VISION PROJECT

SAMPLE ORDER SPEC — “THE VISION CHART” TEE (DROP: EDITION 01)

July 2026 • Send this document to the printer/supplier as-is, together with TVP-EDITION01-PRINT-ARTWORK.pdf and TVP-EDITION01-MOCKUP.png • Contact: Richard (richardmuodilim@gmail.com)

# 1. Garment

| Item | Specification |
| --- | --- |
| Style | Heavyweight boxy tee, slightly oversized, dropped shoulder |
| Fabric | 100% combed ring-spun cotton, heavy jersey, 240-280 GSM |
| Collar | Ribbed, structured (must not curl after wash) |
| Colour | Bone / cream (target: warm off-white, ref #F5F3EE). Acceptable blanks: AS Colour Heavy Tee 4416 in Bone/Ecru, or Stanley/Stella heavy line in Natural / Vintage White, or equivalent 240+ GSM blank |
| Sample size | 1 unit, size M |
| Production (after approval) | 50 units total, numbered 1-50. Sizes S/M/L/XL, split TBC on approval |

# 2. Print — general

| Item | Specification |
| --- | --- |
| Method | Screen print, water-based matte ink. NO glossy plastisol shine. Soft hand feel |
| Ink colour | Black, ref #111111 (near-black, not pure black). One colour, all placements |
| Typeface | Space Mono (set in the supplied vector files — do not substitute) |

# 3. Placement A — back: “THE VISION CHART” (main print)

The back print is designed as an optician's eye test chart (Snellen chart): six rows of text, each row smaller than the one above, separated by thin horizontal rules, with acuity marks (20/200 … 20/20) at the left edge. Centred horizontally on the back. Top of the chart sits 8-9 cm below the back collar seam. Total chart width: 26 cm, height approx 24-25 cm. PRINT DIRECTLY FROM THE SUPPLIED 1:1 VECTOR PDF (page 1) — do not re-set the type. Rows top to bottom:

- Row 1 (20/200): “TV” — bold, cap height approx 6 cm.

- Row 2 (20/100): “NOBODY” — bold, cap height approx 3.3 cm.

- Row 3 (20/70): “SAW US COMING” — bold, cap height approx 1.8 cm.

- Row 4 (20/40): “FOR THE ONES THEY DIDN'T SEE” — regular, cap height approx 0.8 cm.

- Row 5 (20/25): “BUILT FROM NOTHING · IRELAND × ITALY” — regular, cap height approx 0.5 cm.

- Row 6 (20/20): “EDITION 01 · N° ___ / 50 · THE DOOR OPENS WITH ONE WORD” — regular, cap height approx 0.3 cm. This row MUST remain crisply legible at close range — confirm your screen mesh holds a 3 mm cap height cleanly (if not, quote DTG or transfer for this row only). The number field stays BLANK; numbers added per unit at fulfilment.

- Rules: thin horizontal rules (approx 0.3 mm stroke, 23 cm wide) between rows, as in the vector file.

# 4. Placement B — front (left chest)

TVP “V” logo mark, 8 cm tall, positioned on the wearer's LEFT chest, approx 7-8 cm below the shoulder seam. Vector artwork on page 2 of the PDF. Same black ink.

# 5. Placement C — inside back neck

“NEVER FORGET WHERE YOU CAME FROM” — one line, centred inside the garment, 2 cm below the inside collar seam. Width approx 9 cm. Page 3 of the PDF. This print faces the wearer — only they see it.

# 6. Artwork

All artwork is supplied print-ready and 1:1 scale in TVP-EDITION01-PRINT-ARTWORK.pdf (3 pages: back chart / front V / inside neck). If your studio needs a different format (AI/EPS/separations), tell us and we deliver within 48 hours.

# 7. Sample approval checklist (before the 50-unit run)

- Fabric weight and structure: heavy (240+ GSM), boxy drape, no thin-tee feel.

- Print position and row sizes vs the vector file (we will measure).

- Row 6 legibility at close range — the make-or-break detail of this design.

- Matte finish: no shine, soft hand feel.

- Wash test: 3 washes at 30°C — no cracking, no fading, collar holds shape.

- Colour match: garment vs #F5F3EE, ink vs #111111, daylight photo.

# 8. Commercial

- Sample: 1 unit, size M, full print, shipped to Ireland (address on order). Quote sample cost + production unit cost at 50 units + lead times.

- Production trigger: on sample approval only. Target unit cost EUR 8-14 all-in.

TVP — Nobody saw us coming.